# SCA 4.1 Framework and Tooling Research

This document records the research into SCA 4.1 for HH-SDR: what SCA 4.1 requires, which frameworks and tools can provide it, how REDHAWK and GEON Technologies based approaches compare, and how SCA 4.1 would fit the current HH-SDR architecture.

## Contents

- 1. Summary

- 2. SCA 4.1 compared with SCA 2.2.2

- 3. Descriptor requirements

- 4. Framework and tooling options

- 5. GEON sca-jtnc assessment

- 6. REDHAWK assessment

- 7. Applicability to the current architecture

- 8. Integration approach

- 9. Readiness of the current codebase

- 10. Target platform considerations

- 11. Ways to proceed

- 12. Recommendation

- 13. Open questions

- Appendix A. Evidence

## 1. Summary

| Question | Answer |
| --- | --- |
| Can HH-SDR adopt SCA 4.1? | Yes. The HH-SDR changes are small, because the MANET stack, radiod and the radio interface do not depend on any SCA version. |
| Can GEON sca-jtnc be the SCA 4.1 Core Framework? | No. It provides the SCA 4.1 interface definitions, schemas and base classes for components and devices, but not the Core Framework itself: no domain, device or application management, no deployment and no file services. |
| Can REDHAWK be used for SCA 4.1? | No. REDHAWK 2.2 is a complete framework, but for SCA 2.2.2, not SCA 4.1. |
| What blocks SCA 4.1? | Choosing a complete SCA 4.1 Core Framework. |
| What can be done now? | Framework-independent preparation: port interface definitions, SCA 4.1 property kinds and test interface in the SCA model, a node reconfiguration call, and an SCA 4.1 output in the descriptor generator. |
| Should the SCA 2.2.2 work be removed? | No. Most of it is version-neutral and reused under SCA 4.1. The small SCA 2.2.2-specific part should be frozen and retired only when its SCA 4.1 replacement passes tests. |

## 2. SCA 4.1 compared with SCA 2.2.2

The main differences that affect HH-SDR. Items marked (verify) must be confirmed against the SCA 4.1 specification text, which was not reachable during this research.

| Area | SCA 2.2.2 | SCA 4.1 | Effect on HH-SDR |
| --- | --- | --- | --- |
| Component base interface | CF::Resource with LifeCycle, PropertySet, PortSupplier, TestableObject | Separate interfaces combined into a base component: LifeCycle, ControllableInterface (start and stop), PropertySet, PortAccessor, TestableInterface, ComponentIdentifier | Adapter operations are renamed and regrouped |
| Ports | getPort per port | connectUsesPorts, disconnectPorts, getProvidedPorts in bulk | Port handling changes in the adapter |
| Test | runTest(id, values) | runTest(id, values) on TestableInterface; tests declared as test properties in the property file | Test ids become declared properties; the in-tree runTest needs a values parameter |
| Property kinds | configure, execparam, allocation, test, factoryparam | Property kinds revised (verify exact set) | Kind names in the SCA model and descriptors change |
| Profiles | One profile | Several profiles and optional units of functionality | The required profile must be chosen |
| Middleware | CORBA required | CORBA as the defined transfer mechanism, with provision for others (verify) | Decides whether a CORBA runtime must run on the radio |
| Components and devices | Resource and Device | Base component, base device, executable, loadable, aggregate, and component factories | Radio maps to a device; MANET node maps to a component |
| Descriptors | DTDs | DTDs and schemas for SCA 4.1 (Appendix D-1) | New descriptor set required |

## 3. Descriptor requirements

| Descriptor | Purpose | HH-SDR need | Current state |
| --- | --- | --- | --- |
| SPD (software package) | Describes a component’s implementations and code | MANET_Node, Radio device, OpenCPI-derived components | SCA 2.2.2 SPD for MANET_Node, with x86_64 target values |
| SCD (component descriptor) | Interfaces and ports | MANET_Node, Radio device | SCA 2.2.2 SCD for MANET_Node; port interfaces are placeholders with no interface definition |
| PRF (properties) | Properties and kinds | MANET_Node (23 properties), Radio device | SCA 2.2.2 PRF for MANET_Node; property table exists in code |
| SAD (application assembly) | Components, connections, assembly controller | The HH-SDR application | Not written |
| DCD (device configuration) | Devices on a node | Radio device per node | Not written |
| DMD (domain manager) | Domain identity | One per domain | Not written; domain topology undecided |
| Platform and application deployment descriptors | Deployment in SCA 4.1 | If the chosen profile requires them (verify) | Not written |

Descriptor tooling:

| Tool | What it does | SCA version | Status |
| --- | --- | --- | --- |
| ocpi2sca (in-house) | Generates SPD, SCD and PRF from OpenCPI component metadata; validates against schemas | SCA 2.2.2 | Working first version; SCA 4.1 output needed |
| sca-convert (GEON) | Converts REDHAWK component projects to SCA 4.1 | REDHAWK to SCA 4.1 | SPD, SCD and PRF only; no SAD or DCD; needs REDHAWK installed |
| sca-codegen (GEON) | Generates C++ and Python component projects | SCA 4.1 (REDHAWK generator renamed) | Tooling only |
| SCA 4.1 schemas | Validation | SCA 4.1 | Available in sca-jtnc (xml/dtd, xml/xsd) |

## 4. Framework and tooling options

| Option | SCA version | Complete Core Framework? | ARM32 / Yocto | Maintained | Fit |
| --- | --- | --- | --- | --- | --- |
| GEON sca-jtnc | Partial SCA 4.1 | No | Yocto layer tested on rocko only; not on PetaLinux | Dormant since 2019 | Base classes and schemas only |
| REDHAWK 2.2 | SCA 2.2.2 | Yes | meta-redhawk-sdr has a thud branch (the Yocto release under PetaLinux 2019.2) | No active releases | Complete, but not SCA 4.1 |
| sca-jtnc plus in-house Core Framework | SCA 4.1 | After building domain, device and application management, deployment, file services and a parser | Port required | In-house | Large effort, high risk |
| Commercial SCA 4.1 Core Framework | SCA 4.1 | To be evaluated | To be evaluated | Vendor | Needs evaluation: coverage, ARM32 support, memory, licence, cost |
| No framework (current SCA-compatible design) | SCA-compatible, not conformant | No | Native | In-house | Works today; no deployment or interoperability |

## 5. GEON sca-jtnc assessment

sca-jtnc describes itself as an implementation of part of SCA 4.1. It is derived from REDHAWK 2.2, with REDHAWK’s control layer removed and its interfaces moved to the SCA 4.1 definitions.

| Area | Status |
| --- | --- |
| SCA 4.1 interface definitions and schemas | Provided |
| Component base class: initialize, release, configure, query | Implemented |
| Component start and stop | Flag only |
| Port connection | Connect implemented; disconnect and provided-port query empty |
| Test interface | Empty |
| Device base class | Partial: capacity allocation always succeeds; operational state always enabled |
| Executable device | Partial: loads components as libraries into the device process |
| Loadable and programmable devices, FPGA load | Empty, or code generator templates only |
| Aggregate devices | Implemented |
| Domain manager, device manager, application factory, component factory | Not implemented (interface definitions only) |
| File services, installation, deployment | Not implemented |
| Descriptor parsing | Python only; the C++ parser is empty |
| Logging | Not implemented |
| Events | Python only |
| Launching components and devices | Python 2 sandbox only |
| Middleware | CORBA (omniORB); multi-threaded |
| Tests | Five Python 2 sandbox tests, one recorded as failing; no C++ tests; no CI |
| Yocto layer | Depends on meta-redhawk-sdr; tested on rocko; no PetaLinux support; licence field inconsistent |
| Maintenance | Last commit May 2019; no releases |

Conclusion: sca-jtnc is useful as a reference and as a source of the SCA 4.1 interface definitions and schemas. It is not a Core Framework HH-SDR can deploy on.

## 6. REDHAWK assessment

| Aspect | Finding |
| --- | --- |
| SCA version | REDHAWK 2.2 targets SCA 2.2.2, not SCA 4.1 |
| Core Framework | Complete: domain manager, device manager, application factory, node booter, general-purpose processor device |
| Middleware | CORBA (omniORB), multi-threaded |
| Memory | Measured on x86 CentOS 7: framework idle about 37 MiB PSS; about 8 MiB per C++ component; about 20 MiB per Python component |
| Embedded build | meta-redhawk-sdr has a thud branch with omniORB 4.2.3 and REDHAWK 2.2.3; no evidence of a PetaLinux build |
| Role for HH-SDR | An interim SCA 2.2.2 path if SCA 4.1 cannot be met; the base that sca-jtnc was derived from |

## 7. Applicability to the current architecture

| HH-SDR element | SCA 4.1 role | How it fits |
| --- | --- | --- |
| MANET stack (hh_node_*) | Component (MANET_Node) | A thin C++ wrapper calls hh_node_init, configure, start, stop and release. The C stack is unchanged. |
| hh_node_tick control loop | Not an SCA element | Stays the single thread that runs the stack. SCA calls are queued and executed by this loop. |
| radiod and librc | Backing for the Radio device | A thin C++ Radio device is a librc client. It never opens OpenCPI itself. |
| hh_radio_ops_t | Not an SCA element | Stays the hardware seam below the device. |
| OpenCPI application | Not an SCA element | Owned only by radiod. The SCA programmable-device model is not used for the FPGA. |
| MANET data path | Not an SCA element | Stays in C. No CORBA, SCA transport or control socket in the fast path. |
| SCA model (resource.c) | Source of the property file and port list | Property ids, bindings and ports are reused; kinds and the test interface change. |
| BIT | Test interface | BIT checks can be exposed as SCA 4.1 test properties. |

Conflicts and how they are handled:

| Conflict | Handling |
| --- | --- |
| CORBA runs calls on its own threads; the MANET stack is single-threaded | CORBA threads only enqueue; the control loop executes |
| SCA device models load FPGA images; radiod is the only FPGA owner | Radio device is a client of radiod; FPGA loading stays in radiod |
| New dependencies (C++, CORBA runtime) against the project’s no-dependency rule | Confined to the SCA wrapper processes; the MANET stack and radiod stay free of them |
| Memory on the target | Measure the chosen framework on the target before committing |

## 8. Integration approach

Integration architecture

| Boundary | Rule |
| --- | --- |
| SCA boundary | Ends at the two C++ wrappers. Nothing below them uses CORBA. |
| Control boundary | SCA calls are queued and run by the existing control loop. The MANET fast path never waits on SCA. |
| Radio boundary | The Radio device reaches the radio only through librc and radiod. |
| OpenCPI boundary | Only radiod opens the OpenCPI application. |
| Data boundary | MANET traffic never passes through the SCA framework. |

Stages:

- Select the framework. Evaluate complete SCA 4.1 Core Frameworks against the target (ARM32, PetaLinux, about 117 MB usable memory) and record the decision.

- Framework-independent preparation, which can start now:

  - port interface definitions for MANET_Node and Radio;

  - SCA 4.1 property kinds and a test interface with values in the SCA model;

  - a node call for runtime reconfiguration;

  - an SCA 4.1 output in ocpi2sca, beside the SCA 2.2.2 one.

- Wrappers. Build the MANET_Node component and the Radio device against the chosen framework, with the call queue.

- Descriptors. SCA 4.1 SPD, SCD and PRF for both, plus DCD, SAD and DMD.

- Target. Yocto recipes, PetaLinux image, boot order, memory and boot-time measurement.

- Retire SCA 2.2.2 artifacts once their SCA 4.1 replacements pass tests.

## 9. Readiness of the current codebase

| Item | SCA 2.2.2 dependency | SCA 4.1 action |
| --- | --- | --- |
| MANET stack, radiod, librc, radio interface | None | Keep unchanged |
| SCA model (src/sca/resource.c, include/hhsdr/sca/resource.h), 469 lines | About 70 lines: kind names, test interface shape | Modify |
| MANET_Node wrapper (REDHAWK branch): C adapter and 56 tests | Operation names | Reuse as the C side of the new wrapper |
| MANET_Node property file | SCA 2.2.2 document type and kinds | Modify |
| MANET_Node package and component descriptors | SCA 2.2.2 document type, interfaces, x86_64 target | Replace |
| ocpi2sca reader and model | None | Keep |
| ocpi2sca mapping and validation | Kinds, component interface, schema layer | Modify |
| ocpi2sca emitter and vendored schemas | SCA 2.2.2 | Replace, or keep beside a new SCA 4.1 emitter |
| SCA compatibility document | SCA 2.2.2 | Rewrite for the chosen target |

No branch has a build dependency on REDHAWK, CORBA or omniORB.

## 10. Target platform considerations

| Consideration | Finding |
| --- | --- |
| Processor | Zynq-7000, dual Cortex-A9, ARM32. No framework examined needs x86 or 64-bit. |
| Operating system | PetaLinux 2019.2 (Yocto thud). No framework examined has evidence of a PetaLinux build. |
| Memory | About 117 MB usable after the OpenCPI DMA reservation. A REDHAWK-class framework used about 37 MiB idle on x86. Target measurement is required. |
| Init system | BusyBox init. No framework examined requires systemd. |
| Python | sca-jtnc and REDHAWK tooling depend on Python 2. Runtime use of Python should be avoided on the target. |
| Boot time | Target 120 s. The framework adds start-up steps that must be measured. |

## 11. Ways to proceed

| Path | What it means | Effort | Risk | Choose when |
| --- | --- | --- | --- | --- |
| A. Adopt a commercial SCA 4.1 Core Framework | Buy or license a complete framework; build the two HH-SDR wrappers on it | Medium | Medium: cost, licence, ARM32 support | A suitable framework runs on the target within the memory budget |
| B. Build the Core Framework on sca-jtnc | Write the missing domain, device and application management, deployment and parser on GEON’s base classes | Very large | High: dormant base, Python 2, no PetaLinux evidence | No suitable framework exists and SCA 4.1 is mandatory |
| C. Use REDHAWK 2.2 now, move to SCA 4.1 later | Deploy with a complete SCA 2.2.2 framework; keep the design version-neutral for a later SCA 4.1 move | Medium | Medium: two migrations; SCA 2.2.2 only | A working SCA domain is needed soon and SCA 4.1 can wait |
| D. Stay SCA-compatible without a framework | Keep the current in-process SCA model; do only the framework-independent preparation | Small | Low now; defers the decision | SCA 4.1 conformance is not yet a firm requirement |

All four paths share the framework-independent preparation in section 8, stage 2, so it can start now whatever is chosen.

## 12. Recommendation

- Target SCA 4.1, with a staged migration.

- Do not use sca-jtnc as the Core Framework. Use its SCA 4.1 interface definitions and schemas as references.

- Make the Core Framework selection the first task. Evaluate complete SCA 4.1 Core Frameworks for ARM32 PetaLinux and the memory budget.

- Start the framework-independent preparation now (section 8, stage 2).

- Keep the existing SCA work. Freeze the SCA 2.2.2 descriptors and retire them only after their SCA 4.1 replacements pass tests.

- Keep SCA out of the fast path and away from the FPGA. CORBA calls are queued into the control loop; radiod stays the only FPGA owner.

If no complete SCA 4.1 Core Framework fits the target, the choice is between building the missing control layer on sca-jtnc (large effort, high risk) and staying on SCA 2.2.2 with REDHAWK for now. That is a cost and schedule decision.

## 13. Open questions

Decisions for the project:

| # | Question |
| --- | --- |
| 1 | Which SCA 4.1 profile and units of functionality are required? |
| 2 | Is formal SCA 4.1 compliance testing required, or compatibility only? |
| 3 | Which SCA 4.1 Core Framework will be used? |
| 4 | One SCA domain per node, or one across the mesh? |
| 5 | Is CORBA acceptable on the radio, or is another transfer mechanism required? |
| 6 | What memory budget can the SCA framework have? |

Questions for GEON Technologies:

| # | Question |
| --- | --- |
| 1 | Is sca-jtnc maintained, and is there a newer version with domain, device and application management? |
| 2 | Which SCA 4.1 profile was it written for, and was compliance testing attempted? |
| 3 | What launched devices and components outside the Python sandbox? |
| 4 | Has it run on Zynq-7000 ARM32, on which Yocto release, and with what memory use? |
| 5 | Has meta-sca-jtnc been used with PetaLinux? |
| 6 | Which licence applies: the GPL-3.0 in the recipe or the LGPL-3.0 in the code? |
| 7 | Was FPGA loading ever implemented against a real loader? |
| 8 | Is there a Python 3 version, or a plan to remove Python from the runtime? |
| 9 | Does GEON offer a newer or commercial SCA 4.1 Core Framework, and what are its profile, memory use and ARM support? |

## Appendix A. Evidence

sca-jtnc at commit 9059fc2 (May 2019); meta-sca-jtnc at 5eef057.

| Finding | Location |
| --- | --- |
| Implements part of SCA 4.1 | sca-jtnc/README.md:2 |
| C++ descriptor parser is empty | sca-jtnc/base/parser/stub.cpp |
| Capacity allocation always succeeds; operational state constant | sca-jtnc/base/framework/DeviceComponent.cpp:158-167 |
| Test, disconnect and provided-port operations empty | sca-jtnc/base/framework/ResourceComponent.cpp:327-334 |
| Load, unload and terminate empty | sca-jtnc/base/framework/DeviceComponent.cpp:509-522 |
| FPGA load stub in the programmable-device template | sca-jtnc/.../programmable/templates/programmable_base.h:739-776 |
| omniORB and Python 2 required | sca-jtnc/configure.ac:87-114,218-227 |
| Yocto layer tested on rocko only; no compatibility declaration | meta-sca-jtnc/README.md:4; meta-sca-jtnc/conf/layer.conf |
| Recipe licence and compiler flags | meta-sca-jtnc/recipes-core/sca-jtnc/sca-jtnc_4.1.0.bb:22,97 |
| HH-SDR SCA model | src/sca/resource.c, include/hhsdr/sca/resource.h |
| HH-SDR node lifecycle | include/hhsdr/manet/node.h:66-85 |
| MANET_Node SCA 2.2.2 descriptors and adapter | branch feature/redhawk-sca-integration, sca/MANET_Node/ |
| REDHAWK memory measurement | docs/CF-RAM-MEASUREMENT.md |
| Full assessment | temp/SCA41-SCA-JTNC-ASSESSMENT.md |